Clock & Trigger Distribution Board Rev.2, User Manual

| Author | Laboratory |  | Approved by | Laboratory |
| --- | --- | --- | --- | --- |
| Karl-Heinz Sulanke DESY |  |  |  |  |

List of Abbreviations

| 76h | 76 hexadecimal |
| --- | --- |
| CTDB | Clock & Trigger Distribution Board |
| DTB | Digital Trigger Backplane |
| FEB | Frontend Board |
| L2BP | L2 Crate Backplane |
| L2CB | L2 Controller Board |
| LVDS | Low Voltage Digital Signalling |
| MISO | master in slave out |
| MOSI | master out slave in |
| PDB | Power Distribution Box |
| PMT | Photo Multiplier Tube |
| RO | Read Only |
| RW | Read / Write |
| RWC | Read / Write Clear |
| SCLK | serial clock |
| SPI | Serial Peripheral Interface |
| SYNCn | low active synchronization (frame signal) |
| TIB | Trigger Interface Box |
| LED | Light emitting diode |

| History |  |  |
| --- | --- | --- |
| Version | Date | Observation |
| 1 | 05/01/2018 | 1st Revision |
| 2 | 21/05/2020 | monitoring LEDs meaning added |

| Distribution |  |
| --- | --- |

Table of Contents

1.	Introduction	4

2.	Slow Control via SPI Bus	5

3.	FEB Powering and Current Measurement	7

3.1	FEB Powering by Jumper	9

4.	Register Description	10

4.1. PON_TIME Register	10

4.2. POFF_TIME Register	10

4.3. ADC_SRATE Register	10

4.4. DEBUG Register	11

5. Register Address Map	12

6. Register, Bits and Bytes	13

7. Monitoring LEDs	16

# Introduction

This document describes the CTDB, in detail its register functionality. Each CTDB is connected to 15 FEBs via Cat.7 cables. The L2-crate is populated with 18 CTDBs. The slot address (L2-crate position, the CTDB is plugged to) of each CTDB is hard encoded in the L2BP, means there is no need to assign an address e.g. by using jumpers. The CTDB, like the L2CB is powered by an auxiliary power supply, NOT by the main camera power supply being used to power the 265 FEBs. The auxiliary power supply comes up first, ensuring that all FPGAs are loaded, before the software is able to power on the FEBs.

Main tasks of the CTDB are:

- clock and 1PPS distribution to the FEBs, sourced by the L2CB

- reception and propagation of 15 L1 signals

- fanning out and propagating  the camera trigger signal to the FEBs

- reception and propagation of the FEB BUSY signals to the L2CB

The 256 x 16 bit register block is accessible via the SPI bus, physically located on the L2BP. This SPI bus is shared among all 18 CTDBs, populating the L2-crate. Bus master is the L2CB.

Figure 1: Trigger and Power Topology

# Slow Control via SPI Bus

Figure 2: SPI Bus Modes. The used SPI Bus mode is of the type CPOL = 0, CPHA = 1

| L2 Backplane, SPI Bus signals |  |
| --- | --- |
| SPI_SCLK | L2CB output that provides 32 clock cycles at each slow control access. The frequency of the clock it is 6.25 MHZ . The electrical standard for this signal is LVDS. Other frequencies are possible. |
| SPI_SYNCn | L2CB output , low during the slow control access and else high. Electrical standard for this signal is LVDS. |
| SPI_MOSI | Master output / slave input data line. A 32-bit word is transmitted at each slow control access. Electrical standard for this signal is LVDS. The most significant bit is transmitted first. The format of the 32-bit word is shown in table 1. |
| SPI_MISO | Master input / slave output data line. Electrical standard for this signal is LVDS. A 16-bit word is transmitted at each slow control access. The most significant bit is transmitted first. Active at a read access, else tristated. This line provides the information of the addressed CTDB register. |

| Bit range | Usage |
| --- | --- |
| 31 | RD = ‘0’, WR = ‘1’ |
| 30..29 | ‘0’, not used |
| 28..24 | slot address 4..0 (1..9, 13..21) |
| 23..16 | CTDB register address 7..0 |
| 15..0 | CTDB register data 15..0 |

Table 1: SPI bit usage, Bit-31 (MSB) is sent first

| SPI_SYNC. SPI_SCLK. SPI_MOSI. SPI_MISO. Figure 5: Measured, SPI write cycle of 1234h to address 20h of CTDB at slot 2. |  |
| --- | --- |
| SPI_SYNC. SPI_SCLK. SPI_MOSI. SPI_MISO. Figure 6: Measured, SPI read of 1234h from address 20h of CTDB at slot 2. |

Measured SPI bus cycles are illustrated above. The Bus cycle time is about 5.4 us. Higher rates can be achieved by adapting the L2CB firmware accordingly.

# FEB Powering and Current Measurement

To power on the FEBs, one has to set the appropriate bit of the PONF register. The bit position corresponds to the CTDB channel number, with number one for the top and number 15 for the bottom channel. The numbers are engraved at the left side of the RJ45 connectors. Because the CTDB  channel numbering starts with ‘1’,  Bit_0 of the registers PONF, OVER_CUR and UNDER_CUR  is unused.

The firmware continuously measures the current for the 15 FEBs. The default cycle time is 44.8us. The registers CUR_01 to CUR_15 provide a 12 bit current value. To get mA it has to be multiplied with 0.485 resulting in almost 2A for the full range.

The current limits are defined by the CUR_MIN and CUR_MAX registers. Once the current of a particular channel is not within the limits, the power gets shut down immediately and the STAT register bit_0 pops up. By reading both, the OVER_CUR and the UNDER_CUR register, the male functioning channel can be identified easily by the appropriate bit been set. In case of a real shortcut, it is possible that the bit of the UNDER_CUR register is set, instead of the expected bit of the OVER_CUR register. Reason is the power switch circuit FPF2702MX , acting faster than the firmware fuse (max. 44.8us).

To recover from such a state, the appropriate bit of the PONF has to be cleared and set again. This operation clears the corresponding error status bits as well.

| SADC1_nCS |  |
| --- | --- |
| SADC1_SCLK |  |
| SADC1_DIN |  |
| SADC1_DOUT |  |
| Figure 7: SPI signals of the ADC AD7928 (U45, U46), used for the current measurement |  |

Figure 8: power switch circuit FPF2702MX. Figure 9: one of two ADCs AD7928, being used for the current measurement.

## FEB Powering by Jumper

|  | SEL(3) |
| --- | --- |
|  | SEL(2) |
|  | SEL(1) |
|  | SEL(0) |
| Figure 10: Test pins SEL(0) to SEL(3) |  |

The FEBs can also be powered  in groups by setting jumpers on  the test pin field J16, signals SEL(0)...SEL(3), according to the following schema:

| SEL-jumper set |  |  |  | CTDB channel powered |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 2 | 1 | 0 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
|  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |
|  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x | x |
|  |  | x | x |  |  |  |  |  |  |  |  |  |  |  |  | x | x | x |
|  | x |  |  |  |  |  |  |  |  |  |  |  |  |  | x | x | x | x |
|  | x |  | x |  |  |  |  |  |  |  |  |  |  | x | x | x | x | x |
|  | x | x |  |  |  |  |  |  |  |  |  |  | x | x | x | x | x | x |
|  | x | x | x |  |  |  |  |  |  |  |  | x | x | x | x | x | x | x |
| x |  |  |  |  |  |  |  |  |  |  | x | x | x | x | x | x | x | x |
| x |  |  | x |  |  |  |  |  |  | x | x | x | x | x | x | x | x | x |
| x |  | x |  |  |  |  |  |  | x | x | x | x | x | x | x | x | x | x |
| x |  | x | x |  |  |  |  | x | x | x | x | x | x | x | x | x | x | x |
| x | x |  |  |  |  |  | x | x | x | x | x | x | x | x | x | x | x | x |
| x | x |  | x |  |  | x | x | x | x | x | x | x | x | x | x | x | x | x |
| x | x | x |  |  | x | x | x | x | x | x | x | x | x | x | x | x | x | x |
| x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x |

The jumper setting overwrites the software control. The current limitation is fixed at 2A. The DEBUG register value must be 0000h (power up value).

# Register Description

Some registers, located at the top address region, allow additional tuning. Their default settings after power on usually do not need any modification. But if required, there is some room for optimizing or debugging.

## 4.1. PON_TIME Register

This register defines the period of time in ms, after software controlled power on, while the firmware fuse is disabled. Otherwise the firmware fuse would react, once the inrush current is above the programmed max. value of 1.6A (typical).The measurement below shows the typical FEB inrush current (green curve,~2A pk). The current limitation of about 2A here is due to the configuration of the FPF2702MX, see R63 at Figure 8.

| PON_TIME CUR_MAX CUR_MIN |  | POFF_TIME |
| --- | --- | --- |
|  | Voltage Current |  |
| Figure 11: 24V Power on (left) and off (right), measured FEB current and voltage |  |  |

## 4.2. POFF_TIME Register

This register defines the period of time in ms, after software controlled power off, before the next power on is possible. Goal is, to wait until all capacitors are fully discharged and the FEB voltage is almost zero. Keep in mind, the FPGA configuration needs a voltage ramp to get started.

## 4.3. ADC_SRATE Register

The refresh rate for the FEB current ADC can be set by writing a number between 8 and 255 to the ADC_SRATE register. The standard refresh rate is 44.8us  (~22kHz) .

refresh_rate = 5.6us  * ADC_RATE (= 44.8 us after power on)

## 4.4. DEBUG Register

The DEBUG register allows to program the usage of the test pin field J16 (SEL(0)..SEL(3)), see figure 10.  After power up, the test pins can be used as inputs to power a group of FEBs according to the table shown at chapter 3.1. . By writing a value according to the table shown below, the test pins can be used to verify the timing of several signals, e.g. for the backplane SPI bus or the SPI bus used to control the two serial ADCs (FEB current measurement). Note, the pins 1,3,5,7 of J16 are connected to GND.

| DEBUG-reg. value | J16-Pin | Signal (CTDB-schematic) | Remark |
| --- | --- | --- | --- |
| 00h | 2 | SEL(0)-input | Set jumpers to power on groups of FEB w/o any software control. Current limit is fixed at about 2A . |
|  | 4 | SEL(1)-input |  |
|  | 6 | SEL(2)-input |  |
|  | 8 | SEL(3)-input |  |
| 01h | 2 | SADC1_nCS | Check the timing for the current measurement by probing the SPI bus signals of the serial ADC AD7928 (U45). |
|  | 4 | SADC1_SCLK |  |
|  | 6 | SADC1_DIN |  |
|  | 8 | SADC1_DOUT |  |
| 02h | 2 | SADC2_nCS | Check the timing for the current measurement by probing the SPI bus signals of the serial ADC AD7928 (U46). |
|  | 4 | SADC2_SCLK |  |
|  | 6 | SADC2_DIN |  |
|  | 8 | SADC2_DOUT |  |
| 03h | 2 | SPI_SYNC | Check the timing and monitor the SPI bus communication on the L2-crates backplane, with the L2CB acting as the bus master. |
|  | 4 | SPI_SCLK |  |
|  | 6 | SPI_MOSI |  |
|  | 8 | SPI_MISO |  |

# 5. Register Address Map

| hex addr. | Name | Usage |
| --- | --- | --- |
| 00 | PONF | Power On / Off register |
| 01 | CUR_01 | value * 0.485 = FEB_24V_current in mA, RJ45 ports 1..15, |
| ... |  |  |
| 0F | CUR_15 |  |
| 10 | CUR_00 | value * 0.485 = CTDB_(own)_24V_current in mA |
| 11 | CUR_MIN | FEB current, lower limit |
| 12 | CUR_MAX | FEB current, upper limit |
| 13 | OVER_CUR | over current detected |
| 14 | UNDER_CUR | under current detected |
| 20 | CTRL | control register for common usage |
| 21 | STAT | status register for common usage |
| FB | PON_TIME | time in ms, to disable the firmware fuse after power on |
| FC | POFF_TIME | time in ms, after power down, before the next power on is possible |
| FD | ADC_SRATE | ADC (cur. measurement) sampling rate |
| FE | DEBUG | to program the usage of the test pins SEL0..3 |
| FF | FREV | Firmware Revision, bits 15..0 |

# 6. Register, Bits and Bytes

PONF @ address 00h, 16 bit RW, power_on_value = 0 (0000h)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 0 | RO | ‘0’, reserved |
| 15..1 | RW | port_15..1, port_1 is the top connector, write a ‘1’ to power on individual ports, write a ‘0’ to power off the port or to clear the errors status |

CUR_01..15  @ address 01..0Fh, 12 bit RO, power_on_value = 0 (0000h)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 11..0 | RO | FEB current, 0.485 mA / bit, full scale ~2 A |
| 15..12 | RO | ‘0’ |

CUR_00  @ address 10h, 12 bit RO, power_on_value = 0 (0000h)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 11..0 | RO | CTDB-own current load at 24V_aux, 0.485 mA / bit, full scale ~2 A |
| 15..12 | RO | ‘0’ |

CUR_MIN @ address 11h, 12 bit RW, power_on_value = 100 mA (00CEh)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 11..0 | RW | lower current limit, 0.485 mA / bit, full scale ~ 2 A |
| 15..12 | RO | ‘0’ |

CUR_MAX @ address 12h, 12 bit RW, power_on_value = 1600 mA (0CE3h)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 11..0 | RW | uppper current limit, 0.485 mA / bit, full scale ~2 A |
| 15..12 | RO | ‘0’ |

OVER_CUR @ address 13h, 15 bit RO, power_on_value = 0 (0000h)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 0 | RO | ‘0’, reserved |
| 15..1 | RO | ports 15..1, ‘1’ = individual current above the max. current limit |

UNDER_CUR @ address 14h, 15 bit RO, power_on_value = 0 (0000h)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 0 | RO | ‘0’, reserved |
| 15..1 | RO | ports 15..1, ‘1’ = individual current below the min. current limit |

CTRL @ address 20h, 16 bit RW, power_on_value = 0 (0001h)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 0 | RW | firmware fuse enable (check, if FEB current is within the limits) |
| 14..1 | RW | reserved |
| 15 | RW | LED_ENA, ‘1’ enables monitoring LEDs |

STAT @ address 21h, 16 bit RO, power_on_value = 0 (0000h)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 0 | RO | over or under current for at least 1 of the 15 ports detected |
| 1 | RO | if set, current values are available, “dummy”, is on 20us after FPGA load |
| 15..2 | RO | ‘0’, reserved |

PON_TIME @ address FBh, 8 bit RW, power_on_value = 0032h (50ms)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 7..0 | RW | period of time, after power on, while the firmware fuse is disabled, multiples of 1ms |
| 15..8 | RO | ‘0’ |

POFF_TIME @ address FCh, 8 bit RW, power_on_value = 003Ch (60ms)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 7..0 | RW | period of time, after powering off, preventing power up while capacitors at the FEB are not yet fully discharged |
| 15..8 | RO | ‘0’ |

ADC_RATE @ address FDh, 8 bit RW, power_on_value = 0 (0008h)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 7..0 | RW | FEB current ADC, refresh rate, only number >=8 will be accepted, standard min refresh rate is 44.8us refresh_rate = 5.6us * ADC_RATE (= 44.8 us after power on) |
| 15..8 | RO | ‘0’ |

DEBUG @ address FEh, 8 bit RW, power_on_value = 0 (0000h)

| Bit | Attr. | Usage |
| --- | --- | --- |
| 7..0 | RW | 00h : power on groups of FEBs by setting jumpers 01h : serial ADC1 (U45), monitoring SPI bus signals at test pin field J16 02h : serial ADC2 (U46), monitoring SPI bus signals at test pin field J16 03h : L2-crate backplane, monitoring SPI bus signals |
| 15..8 | RO | ‘0’ |

FREV @ address FFh, 16 bit RO, power_on_value : binary firmware rev. number

| Bit | Attr. | Usage |
| --- | --- | --- |
| 11..0 | RO | CTDB firmware revision |
| 15..12 | RO | CTDB PCB revision |

# 7. Monitoring LEDs

Enable the monitoring LEDs for test purposes only, to avoid light emission in the camera! The table shows the meaning of the LEDs. The LEDs can be hard enabled, by setting the jumper J10 or by software, setting bit_15 of the CTRL register.

| LED | color | meaning |
| --- | --- | --- |
| V129 (lower) | red | FPGA configuration failed |
| V130 | green | FPGA configuration successful |
| V132 | yellow | L1 trigger (up going) |
| V131(upper) | blue | L1A camera trigger (down going) |