# Basic Parts

- 1x 	subrack 6HE 84TE 235T LD				24563-452

- 1x	19" bracket type F with handel holes		24564-498

- 1x	front handle, 12.2mm, clear anodised, 6U		10501-008

- 2x	back cover plate (half width), 6U			20848-139

# Tools

- Torx- wrench  T20H

- Slotted screwdriver

# Mounting

## Crate

- mount module rails (front, middle, back)  using screws M4x14/10 (according to „Subrack User Manual 64560-114“  p. 2) to one side wall

- slide in threaded stripes and hole striped

- use right orientation of the side walls, pressed-in holding point inside

- mount handles

- follow the received construction manual !

## Guide Rails and Front Plates

| Gray Guide Rail at Pos. |  |
| --- | --- |
| 1 | 00 – 03 |
| 2 | 04 – 07 |
| 3 | 08 – 11 |
| 4 | 12 – 15 |
| 5 | Red Guide Rail at Pos.. 1; 40 – 43. 16 – 19 |
| 6 | 20 – 23 |
| 7 | 24 – 27 |
| 8 | 28 – 31 |
| 9 | 32 – 35 |
| 10 | 48 – 51 |
| 11 | 52 – 55 |
| 12 | 56 – 59 |
| 13 | 60 – 63 |
| 14 | 64 – 67 |
| 15 | 68 – 71 |
| 16 | 72 – 75 |
| 17 | 76 – 79 |
| 18 | 80 – 83 |

- Mount front plates at not used positions:

  - 36 – 39

  - 44 – 47

## Cover Plates

- Mount cover plates according to „Subrack User Manual 64560-114“

## Back Plane Mounting

# Material needed

- 84x	M2,5x10mm screws

- 84x 	M2,5 washers

- 38x 	M4x10mm screws, brass

- 38x	M4 washers, brass

- 38x	M4 toothed locked washer, brass

- 8x distant bolts, nylon,  M3x15

- Backplane L2BP1

- Copper rails (check dedicated folder)

- 8x 0.2m braided cable  (black, 35mm²) with tubular lugs M8 at both ends

- 0.4m braided cable  (red, 2,5mm²) with tubular lug M4 / wire end ferrule

- 0.4m braided cable  (blue, 2,5mm²) with tubular lug M4 / wire end ferrule

- 0.1m braided cable  (blue, 2,5mm²) with tubular lug M4 / tubular lug M4

- 1x prepared half back plate with 24V power connectors

- 1x prepared half back plate with 240VAC  power connector

# Mounting

- mount backplane L2BPx to the module rails

- mount 8 nylon distant bolts to the copper rails

- mount the copper rails to the backplane using M4 screws / washers (brass)

- attach the black power cables loosely to the copper rails

- attach the power cable and the chassis ground to the back plates power connectors

- Fix the inner power cable connection first

- Fix the outer power cable connection afterwards

- Mount the isolating FR4 plates

- Mount the back plate

- Connect the blue (-), the red (+) of the auxiliary power supply and the chassis ground to the backplane

- Close the back side of the crate. Done !