L0_TB_V10, software commands

Firmware location:

https://www-zeuthen.desy.de/~sulanke/Projects/CTA/Dig_trigger/NectarCam/L0_TB/firmware/

Commands can be in lower case or upper case, spaces may be omitted.

## command table

| command | parameter description | remark |
| --- | --- | --- |
| DAC Bx Cy Dzzz | x = 1..9 -- board address (8 bit switch) y = 0..7 -- DAC channel zzz = 0..255 – decimal DAC data value (8 bit) | Discriminator threshold in mV |
| VER Bx | x = 1..9 -- board address (8 bit switch) | Check if board x is communicating |
| FWR Bx | x = 1..9 -- board address (8 bit switch) | Check firmware rev. of board x |
| BWR Bx Ayy Dzz | x = 1..9 -- board address (8 bit switch) y = 0..7F -- hex register address z = 0..FF – hex register value (8 bit) | Backplane register write via SPI master access |
| BRD Bx Ayy | x = 1..9 -- hex board address (8 bit switch) y = 0..7F --hex register address | Backplane register read via SPI master access |