Digital Trigger Backplane, Firmware

| Author | Laboratory |  | Approved by | Laboratory |
| --- | --- | --- | --- | --- |
| Karl-Heinz Sulanke DESY |  |  |  |  |

| List of Abbreviations |  |  |  |
| --- | --- | --- | --- |
| ATBP | Analog Trigger Backplane | PLL | Phase Locked Loop |
| DTB | Digital Trigger Backplane |  |  |
| FEB | Frontend Board |  |  |
| SPI | Serial Peripheral Interface |  |  |
| FPGA | Field Programmable Gate Array |  |  |
| TIB | Trigger Interface Board |  |  |
| CTDB | Clock & Trigger Distribution Board |  |  |
| 1PPS | 1 Pulse Per Second |  |  |
| TOT | Time Over Threshold |  |  |

| History |  |  |
| --- | --- | --- |
| Version | Date | Observation |
| 01 | 18/05/2020 | Draft |

| Distribution |  |
| --- | --- |

Table of Contents

1	Introduction	3

2	FPGA	3

3	Firmware Description	3

3.1	Clock and 1PPS Generation	3

3.2	Register Block	3

3.3	L0 Delay Tuning	3

3.4	Trigger Logic	3

3.5	DTB, Trigger Firmware, Block Diagram	4

# Introduction

This document describes the basic functionality of the DTB’s firmware, valid for firmware Rev. DTB4_3nn_015.

# FPGA

The FPGA of the type Xilinx XC6SLX16-3CSG324I  is the heart of the Digital Trigger Backplane. After power on, it gets loaded from a serial Flash (AT45DB081E-SSHN). The latter can be updated via a JTAG connection, controlled by the FEB or external Xilinx programming hardware (e.g. Platform Cable USB II).

# Firmware Description

## Clock and 1PPS Generation

The FPGA receives a 50MHz clock, provide by one of the CTDBs, being plugged to the L2-Crate. Every second exactly one clock pulse is modulated, using the 1PPS signal, provided by the TIB. The modulation is achieved by changing the clocks duty cycle from 50/50 to 25/75. The duty cycle change is not affecting the FPGA internal PLL, being synchronized by the leading edge of the clock. A phase shifted clock output of the PLL is used to recover the 1PPS signal, needed by the FEB.

The trigger logic is driven by a 125MHz clock (PLL output).

## Register Block

24 8-Bit register allow various control and calibration procedures. The address range is 7bit. If needed, up to 104 registers can be added.  These register can be accessed via the FEB using the SPI bus. Implemented is a SPI slave interface. For details, see the DTB_user_manual_xx.pdf .

## L0 Delay Tuning

The trigger logic processes the L0 (TOT) signals of 37 pixels. An accurate delay tuning for each of the pixels is required. The presently used Firmware (Rev. 015) allows to program a coarse delay between 0 to 7ns in steps of 1ns and a fine delay of 1ns  in steps of  37ps.

## Trigger Logic

The trigger factory receives the delay-tuned, possible masked and according to the trigger window size extended L0 signals. The CTRL register allows the selection of one of four trigger algorithms:

The trigger 3NN generates a trigger, if 3 connected pixels do exist within the 37 pixel area.

The trigger 1_of_7 generates a trigger, if one of the central cluster pixel fires, useful for FEB-tests.

The trigger 2_of_37 can be used for L0 delay calibration. First the delay of the central pixel is set to a medium delay of 4ns. Then, one of the surrounding 36 pixels can be selected, using the PIXEL_SEL register.  By varying the L0 delay value and analyzing the trigger rate one can find the optimum delay value for the chosen pixel. This can be done for each pixel.  It might be useful to adjust the trigger window size to 1ns before.

The trigger 1_of_37 can be used to check, if any of the 37 L0 signals is available. The PIXEL_SEL register must be used to select the pixel to be checked.

The trigger logic consists out of 8 identical blocks, each processing a time slice of 1ns. The synchronous trigger output signal, generated by an OSERDES stage has a time resolution of 1ns.

## DTB, Trigger Firmware, Block Diagram