ICM datasheet

| Author | Laboratory |  | Approved by | Laboratory |
| --- | --- | --- | --- | --- |
| Karl-Heinz Sulanke DESY |  |  |  |  |

| List of Abbreviations |  |  |  |
| --- | --- | --- | --- |
| PCB | Printed Circuit Board |  |  |
| ICM | IceCube Communication Module |  |  |
| USART | Universal Synchronous/Asynchronous Receiver/Transmitter |  |  |
| MCU | Micro Controller Unit |  |  |
| FPGA | Field Programmable Gate Array |  |  |
| MB | Main Board |  |  |
| GPIO | General Purpose Input / Output |  |  |
| INTLK | Interlock |  |  |
| WP | Wire Pair |  |  |
| PCA | Penetrator Cable Assembly |  |  |

| History |  |  |
| --- | --- | --- |
| Version | Date | Observation |
| 1.0 | 2019-01-24 | Draft |
| 1.1 | 2019-10-22 | corrected PCA_ID voltage level |
| 1.2 | 2020-01-17 | P1, P2, Signal names harmonized with D-Egg / PDOM schematics |
| 1.3 | 2020-05-06 | Block Diagram corrected, signals CAL_trig, CAL_time connected to P1 |
| 1.4 | 2021-01-04 | IceCube upgrade logo used now |
| 1.5 | 2024-04-12 | ICM_1 to ICM_4 differences and test point description added |

Table of Contents

1	Introduction	4

2	Block Diagram	4

3	PCB	5

4	Connector Types	6

5	Signal Transformer	6

6	Connector Signals	6

7	Signal Description	7

7.1	Connector P1, Signals	7

7.2	Connector P2, Signals	8

8	ICM_1 to ICM_4, Differences	9

9	ICM_x Test Points	10

9.1	Communcation, unfiltered Wire Pair and filtered ADC Input signal	10

9.2	Power Supply	11

List of Figures

# Introduction

The document is based on a proposal by Perry Sandström. The idea is to use a unique piggy back module for all in ice devices. The main functionality is communication and time calibration.

# Block Diagram

# PCB

| size | 65 mm x 35 mm |
| --- | --- |
| thickness | 1.6 mm |
| layers | 8 |
| mounting holes | x,y = (32.5, 3.5) + (32.5, 31.5) 2.9 mm drill diameter, metallized, 6 mm top / bottom ring, on GND |
| standoff | 4-40 x 6.35 mm, soldered to mainboard, e.g. Keystone 4883 + screw Keystone 9400 or Wuerth 9774065151 + screw M2.5x5mm, e.g. ETTINGER 01.64.212 |

mounting holes

top view

bottom side, viewing into P1, P2

# Connector Types

| P1, P2 | x,y = (61.5, 17.5) + (4.5, 17.5), Samtec TFM-115-02-S-D-A-K-TR, mates to Samtec SFM-115-02-S-D-A-K-TR (on mainboard) |
| --- | --- |
| J3 (optional) | on bottom side, micro USB, terminal and power connection (standalone mode), Hirose ZX62R-B-5P(30) |
| J4 (optional) | Xilinx Platform Cable USB II Molex 87832-1420 |

# Signal Transformer

| L14 | Coilcraft SWB3010-SMLB |
| --- | --- |

# Connector Signals

| P2 |  |  |  |  | P1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| signal | pin | pin | signal |  | signal | pin | pin | signal |
| GND | 1 | 2 | FPGA_CLOCK_P |  |  | 1 | 2 | WP_ADR_0 |
| FPGA_CLOCK_N | 3 | 4 | GND |  | WP_P | 3 | 4 | WP_ADR_1 |
| FPGA_SYNC_N | 5 | 6 | FPGA_SYNC_P |  | WP_N | 5 | 6 | WP_ADR_2 |
| GND | 7 | 8 | FPGA_GPIO_0 |  |  | 7 | 8 |  |
| FPGA_GPIO_1 | 9 | 10 | GND |  | USB_D_P | 9 | 10 | USB_P5V |
| MCU_USART_CK | 11 | 12 | MCU_nRST |  | USB_D_N | 11 | 12 | P1V8 |
| FPGA_UART_TX | 13 | 14 | FPGA_UART_RX |  |  | 13 | 14 | TDI |
| FPGA_UART_RTS | 15 | 16 | FPGA_UART_CTS |  | CAL_TRIG_P | 15 | 16 | TDO |
| GND | 17 | 18 | MCU_BOOT |  | CAL_TRIG_N | 17 | 18 | TCK |
| MCU_USART_TX | 19 | 20 | MCU_USART_RX |  | CAL_TIME_P | 19 | 20 | TMS |
| MCU_USART_RTS | 21 | 22 | MCU_USART_CTS |  | CAL_TIME_N | 21 | 22 | P1V8 |
| MBPWR_EN | 23 | 24 | INTLK_0 |  | GND | 23 | 24 | GND |
| INTLK_1 | 25 | 26 | INTLK_2 |  | GND | 25 | 26 | GND |
| INTLK_3 | 27 | 28 | PCA_ID |  | P3V3 | 27 | 28 | P5V_IN |
| GND | 29 | 30 | GND |  | P3V3 | 29 | 30 | P5V_IN |

# Signal Description

## Connector P1, Signals Name; Direction; Voltage Level; Remark. WP_P; bidir.; analog; AC (comm.) part of the in ice quads wire pair signal. WP_N. WP_ADR_0..2; in; 3.3V; DOM address select bits, 10K pullups to 3.3V, B”111” activates the 150 ohm line termination, for address decoding connected to the FPGA as well. USB_D_P; bidir; digital; USB connector signals, can be used for a USB-B connector, placed on the mainboard. USB_D_N. USB_P5V; in; 5V; +5V by the USB master, used to power the USB to UART bridge chip (FT2322RQ). TDI; in; 1.8V LVCMOS; FPGA JTAG signals, connect (optional) to a 2x7 pin, 2mm pitch, vertical box header on the mainboard, according to the XILINX Platform Cable USB II definition. TDO; out. TMS; in. TCK; in. P1V8; out; +1.8V by onboard DC/DC, 500 mA max. load, voltage reference for the XILINX Platform Cable. CAL_TRIG_P; out; 1.8V LVDS signal; Trigger pulse. CAL_TRIG_N; out. CAL_TIME_P; out; 1.8V LVDS signal; Timer synchronization pulse. CAL_TIME_N; out. P3V3; out; 3.3V; +3.3V by onboard DC/DC, 500 mA max. load. P5V_IN; in; 5V; +5V by mainbord DC/DC average current : TBD (< 200 mA expected) peak current : TBD. GND; mainboard ground, P5V_IN return.

## Connector P2, Signals Name; Direction; Voltage Level; Remark. FPGA_CLOCK_P; Out; 1.8V LVDS; 20MHz, 0.4V diff., Vocm=0.9V, use 100 ohm termination, receiver can also use LVDS_25 inputs. FPGA_CLOCK_N. FPGA_SYNC_N; Out; 1.8V LVDS; 0.46 diff., Vocm=0.9V, use 100 ohm termination, receiver can also use LVDS_25 inputs. FPGA_SYNC_P. FPGA_GPIO_0; Bidir; 1.8V; general purpose I/O, can also be used as diff., connect to mainboard FPGA. FPGA_GPIO_1. MCU_USART_CK; Out; 1.8V LVCMOS; MCU USART clock. MCU_nRST; Out; 1.8V LVCMOS; MCU low active reset. FPGA_UART_TX; In; 1.8V LVCMOS; connect to mainboard FPGA, signal direction by ICM FPGA view. FPGA_UART_RX; Out. FPGA_UART_RTS; In; buffer status. FPGA_UART_CTS; Out; buffer status. MCU_BOOT; Out; 1.8V LVCMOS; connect to STM32H743 boot pin. MCU_USART_TX; In; e.g., connect to STM32H743 USART3_TX, pin 46. MCU_USART_RX; Out; e.g., connect to STM32H743 USART3_RX, pin 47. MCU_USART_RTS; In; e.g., connect to STM32H743 USART3_TX, pin 53. MCU_USART_CTS; Out; e.g., connect to STM32H743 USART3_TX, pin 52. MBPWR_EN; Out; 3.3V LVCMOS; mainboard power enable (on / off). INTLK_0; Out; 3.3V LVCMOS; Interlock 0...3. INTLK_1; Out. INTLK_2; Out. INTLK_3; Out. PCA_ID; Bidir; 3.3V open drain; connect to 1-wire mainboard ID EEPROM.

# ICM_1 to ICM_4, Differences

| item | ICM_1 | ICM_2 | ICM_3 |  | ICM_4 | remark |
| --- | --- | --- | --- | --- | --- | --- |
| JTAG interface, J4 TCK pull-up resistor | 330R | 330R | 1k |  | 1k | The low-cost programmer JTAG-HS3 has very weak drivers. The config. partly failed, when additional parts (mainboard-FPGA) were in the JTAG chain, due to the too low 330R value at ICM_1, _2. |
| FPGA input PUDC_B | GND | GND Check if your MBPWR_EN has a pull-down of ~1K (330uA x 1k = 0.33V) | P1V8 |  | P1V8 | GND: all I/Os with weak pull-up while configuration See below min. / max. pull-up current vs. VCCO: P1V8, (for ICM_3, _4 mounting option, R48=DNL) all I/Os are floating while configuration |
| Con. P1, pins 15,17,19,21 Signal names and termination scheme | MB_GPIO_0..3 MB_GPIO_0/_1 MB_GPIO_2/_3 by 3.3V FPGA bank_ 34, Each signal with 47R serial termination | CAL_TRIG_P/N, CAL_TIME_P/N Driven by 1.8V FPGA bank_14 Each pair with 100R parallel termination |  |  |  | For ICM_1 other FPGA pins are being used !!! When using ICM_2..4 in the mini_Fieldhub, R81, R82 have to be removed before! |
| Flash write protection Signal D02_nWP | Permanently pull-up to P1V8 by R16 (4k7), pull down to GND per Jumper J6 |  |  | Permanently pulled down by R16 (4k7), C85 (100nF) to P1V8 for debouncing. ICM_4 allows to readback the D02_nWP signal via FPGA pin D10 |  | Use J6 to connect an infrared photo-transistor, e.g. 1540601NEA200. (1540601NEA200 is hand-soldered on J6 pads) Disable the write protection using an infrared lamp (~900nm). |
| Signal PCA_ID (MB_ID) | MB_ID w/o pull-up | PCA_ID with 1k pull-up (R83) |  |  |  | Not sure why the name got changed to PCA_ID |
| Comm. ADC, diff. anlog input filter | 2x(2x47R+100pF) | 1x (2x470R + 47pF) |  |  |  | Check individual ICM_1 for possible changes |

# ICM_x Test Points

## Communcation, unfiltered Wire Pair and filtered ADC Input signal

Best, use a differential probe. Or two single ended probes + GND connection in differential math-mode.

COM_ADC_INM

COM_ADC_INP

COM_WP_P

COM_WP_N

GND

GND

GND

COM_WP_P

COM_WP_N

COM_ADC_INP

COM_ADC_INM

J10

J2

E.g., the half duplex comms. signal, measured at J2 at an mDOM-ICM, using a 1.5m 100ohm twisted pair cable and a diff. probe (Tektronix P6247).

miniFH

mDOM

mDOM, zoomed

## Power Supply

See the test points below. Use a multimeter or the scope.

GND

GND

GND

1.8V

3.3V

1.0V

5V