# ICM Signals, redefined, to be used for mini_Fieldhub

## ICM#0, Connector P1, power, analog & 3.3V Signals

| pin | direction | new name | old name | Remark |
| --- | --- | --- | --- | --- |
| 3 | bidir | WP0_COM_P | COM_WP_P | analog |
| 5 | bidir | WP0_COM_N | COM_WP_N | analog |
| 2 | in | nCBL_PLGD | MB_ADR_0 |  |
| 4 | out | nTX_LED0 | MB_ADR_1 |  |
| 6 | out | nRX_LED0 | MB_ADR_2 |  |
| 15 |  | USB1_RXD0 | CAL_TRIG_P | R82 must be removed !!! |
| 17 |  | USB1_CTS0 | CAL_TRIG_N |  |
| 19 |  | USB1_TXD0 | CAL_TIME_P | R81 must be removed !!! |
| 21 |  | USB1_RTS0 | CAL_TIME_N |  |
| 23 |  | GND | GND |  |
| 24 |  | GND | GND |  |
| 25 |  | GND | GND |  |
| 26 |  | GND | GND |  |
| 28 | in | P5V | P5V_IN |  |
| 30 | in | P5V | P5V_IN |  |
| 27 | out | ICM_P3V3 | P3V3 |  |
| 29 | out | ICM_P3V3 | P3V3 |  |

## ICM#0, Connector P2, 1.8V Signals

| pin | direction | new name | old name | Remark |
| --- | --- | --- | --- | --- |
| 2 | in | ICM1_CLOCK_P | FPGA_CLOCK_P |  |
| 3 | in | ICM1_CLOCK_N | FPGA_CLOCK_N |  |
| 6 | in | ICM1_SYNC_P | FPGA_SYNC_P |  |
| 5 | in | ICM1_SYNC_N | FPGA_SYNC_N |  |
| 9 | out | ICM0_CLOCK_P | FPGA_GPIO_1 |  |
| 8 | out | ICM0_CLOCK_N | FPGA_GPIO_0 |  |
| 11 | out | ICM0_SYNC_P | FPGA_GPIO_3 |  |
| 12 | out | ICM0_SYNC_N | FPGA_GPIO_2 |  |
| 13 | in | ICM1_TSTRG | FPGA_UART_TX |  |
| 14 | out | ICM0_TSTRG | FPGA_UART_RX |  |
| 15 | out | WP0_ADC_nCS0 | FPGA_UART_RTS |  |
| 16 | out | WP0_ADC_nCS1 | FPGA_UART_CTS |  |
| 18 | in | WP0_ADC_DOUT0 | STM_BOOT |  |
| 19 | in | WP0_ADC_DOUT1 | STM_USART_TX |  |
| 20 | out | WP0_ADC_SCLK | STM_USART_RX |  |
| 21 | in | ICM0_nMASTER | STM_USART_RTS |  |
| 22 |  |  | STM_USART_CTS |  |

## ICM#0, Connector P2, 3.3V Signals

| pin | direction | new name | old name | Remark |
| --- | --- | --- | --- | --- |
| 23 | out | WP0_nPON | INTLK_ENA | open drain |
| 24 | in | ZYNQ_RXD0 | INTLK_0 |  |
| 25 | out | ZYNQ _CTS0 | INTLK_1 |  |
| 26 | out | ZYNQ _TXD0 | INTLK_2 |  |
| 27 | in | ZYNQ _RTS0 | INTLK_3 |  |
| 28 |  |  | MB_ID |  |

## ICM#0, Connector P2, GND Signals

| pin | direction | new name | old name | Remark |
| --- | --- | --- | --- | --- |
| 1 |  | GND | GND |  |
| 4 |  | GND | GND |  |
| 7 |  | GND | GND |  |
| 10 |  | GND | GND |  |
| 17 |  | GND | GND |  |
| 29 |  | GND | GND |  |
| 30 |  | GND | GND |  |