# SpoolHub + BatteryBox, Shipping List, Spole 25/26

| items | Qty. | remark |
| --- | --- | --- |
| Battery box | 1 | Incl. custom front panel |
| Battery chargers | 2 | Mounted to battery-box, Bauer Electronics, 48V, LiFePO4, 58,4V 10A 600W |
| Heating Mats | 2 | 52.5x25 cm, 50W, with thermostat, |
| SpoolHub | 1 |  |
| Assembled PCB “FSEB_4” | 9 | Custom made |
| 125VAC NEMA-15 power cord | 1 | For in-lab tests, required to charge the batteries |
| Standard quad patch cables | 3 | 12 pin to 12pin, 2m |
| Paro patch cable | 1 | RS485 + power, 5 pin to 5pin, 2m |
| LOM logger cable | 1 | 12 pin to 5 pin, AUX quad, 2m |
| Power cable, shielded | 1 | Slipring to SpoolHub, 5 pin to 5pin, 2m |
| Power cable, shielded | 1 | BatteryBox to slipring, 5 pin to 5pin, 3m |
| Standard quad to DSUB9f adapter | 1 | Connecting mainboards for test purposes |
| LOM logger to DSUB9f adapter | 1 | Connecting mainboards for test purposes |
| Neodym magnets, d=32mm | 8 | 45kg, incl. eye-bolts, for temporary cable fixing on the spool |

# SpoolHub + BatteryBox, List of Spares, Spole 25/26

| items | Qty. | Manufacturer ID | remark |
| --- | --- | --- | --- |
| Assembled PCB “FDOR_3” | 1 |  | Spoolhub spare |
| Assembled PCB “FBAP_mini” | 1 |  | Spoolhub spare |
| 125VAC power cord, custom, 2m | 1 |  | U.S. standard, NEMA-15 |
| Capacitor 10uF, 100V, 2220 | 6 |  | spares |
| Thermistor, 2R | 3 |  | spares |
| Thermistor, 5R | 2 |  | spares |
| Fuse holder, 20A, 250V | 1 | 03453LF1H | For fuse size 0.25" x 1.25" |
| Fuses, see below |  | Fuse, Cartridge, Time Delay, xx A, 250 VAC, 0.25" x 1.25" |  |
| Fuse 2A, slow | 5 | MP007106 | SpoolHub +/-50V outputs |
| Fuse 5A, slow | 5 | MP007105 | Possible replacement for the MP007106 |
| Fuse 20A, 500V, high inrush | 3 | 0ADGC9200-BE | BatteryBox, 125VAC input |
| Fuse 25A, 500V, high inrush | 3 | 0ADGC9250-BE | Possible replacement for 20A |
| Fuse 30A, 500V, high inrush | 3 | 0ADGC9300-BE | Possible replacement for 20A |
| Pilot lamp red | 1 | E1316 | LED lamp |
| Pilot lamp green | 1 | E1318 | LED lamp |
| NEMA 5-15 cable socket | 2 | 15W47MB | 15A, 125VAC |
| NEMA 5-15 cable plug | 2 | 14W47 | 15A, 125VAC |
| USB 2.0 cable A to B, 0.5m | 2 | 84809 | SpoolHub-internal |
| Ethernet CAT6A patch cable, 0.5m | 1 | TG 100007966 | SpoolHub-internal |
| Shielded cable, 2 pairs, d=6mm | 10m | LAPP 1032108 |  |
| cable, 2 pairs, d=4mm | 2m | LAPP 912023 |  |
| cable, 1 pair, d=3mm | 1m | LAPP 91200/1 |  |
| Stranded wire, silicone, 1.5mm2 | 1 |  | Red |
| Stranded wire, silicone, 1.5mm2 | 1 |  | Yellow / Green |
| Stranded wire, silicone, 1.5mm2 | 1 |  | Blue |
| Stranded wire, silicone, 1.5mm2 | 1 |  | black |
| Connector | 1 | PT02E-14-5S |  |
| Connector | 1 | PT02E-14-5P |  |
| Connector | 1 | PT01A-14-5P(SR) |  |
| Connector | 1 | PT01A-14-5S(SR) |  |
| Connector | 1 | PT06E-14-12P(SR) |  |
| Connector | 1 | PT01A-14-12S(SR) |  |
| Connector | 1 | PT06A-14-5P(SR) |  |
| Connector incomplete | 1 |  | Spares, inch screws+ |
| Connector cover | 2 | MS31811CA | SpoolHub, BatteryBox |
| Connector sealing gasket,14” | 2 | MS52000-4 | SpoolHub, BatteryBox |
| Connector DSUB9f, solder caps | 2 |  |  |
| Connector DSUB9m, solder caps | 2 |  |  |
| DSUB9 shell | 2 |  |  |
| Ring terminals, M8 | 6 |  | Battery connection |
| Ring terminals, M4 | 2 |  |  |
| Ring terminals, M3 | 2 |  |  |
| WAGO terminal, 5 way | 1 |  | BatteryBox |
| WAGO terminal, 3 way | 2 |  | BatteryBox |
| Gluing pads for cable ties | 6 |  | BatteryBox |
| Cable ties, 11cm | 10 |  | BatteryBox |
| Cable ties, 5cm, Nylon | 10 |  | SpoolHub internal |
| Screw M5x12, countersunk, Nylon | 8 |  | SpoolHub brackets |
| Washers M5 Nylon | 12 |  |  |
| Screw M4x12, countersunk, Nylon | 20 | MP008100 | SpoolHub magnets |
| Screws, washers, M2.5 |  |  | Various |
| Screws, washers, M3 |  |  | Various |
| Screws, washers, M4 |  |  | Various |